# DLD Assignment 3 - Complete Solutions

National University of Computer & Emerging Sciences, Karachi
Spring 2026 FAST School of Computing
Solution Date: 28-April-2026

## Problem 1: Active-Low S-R Latch

If the waveforms in Fig-1 are applied to an active-Low S-R latch, draw the resulting Q output waveform in relation to the inputs. Assume that Q starts Low.

### Solution 1:

For an active-LOW S-R latch:

• S = 0 (active), R = 1 (inactive): Q = 1 (Set)
• S = 1 (inactive), R = 0 (active): Q = 0 (Reset)
• S = 1, R = 1: Q holds previous state (No change)
• S = 0, R = 0: INVALID state (forbidden)

Timing Diagram Description:

• When S goes LOW while R stays HIGH: Q transitions to HIGH
• When R goes LOW while S stays HIGH: Q transitions to LOW
• When both S and R are HIGH: Q maintains its previous state
• Initial condition: Q = LOW (0)

## Problem 2: Active-High S-R Latch

Solve problem 1 for the input waveform in Fig-2 applied to an active-High S-R latch.

### Solution 2:

For an active-HIGH S-R latch:

• S = 1 (active), R = 0 (inactive): Q = 1 (Set)
• S = 0 (inactive), R = 1 (active): Q = 0 (Reset)
• S = 0, R = 0: Q holds previous state (No change)
• S = 1, R = 1: INVALID state (forbidden/metastable)

Key Difference from Active-Low:

• Logic levels are inverted
• Set operation requires S=1 instead of S=0
• Reset operation requires R=1 instead of R=0

## Problem 3: Gated S-R Latch

For a gated S-R latch, determine the Q and Q' output for the inputs in Fig-4. Show them in proper relation to the enabled input. Assume that A starts LOW.

### Solution 3:

Gated S-R Latch Operation:

• When ENABLE = 0: Latch is disabled, Q and Q' maintain their previous states
• When ENABLE = 1: Latch acts as a normal S-R latch based on S and R inputs

Truth Table:

| ENABLE | S | R | Q |
| --- | --- | --- | --- |
| 0 | X | X | No Change |
| 1 | 1 | 0 | 1 (Set) |
| 1 | 0 | 1 | 0 (Reset) |
| 1 | 0 | 0 | Hold |
| 1 | 1 | 1 | Invalid |

## Problem 4: Gated D Latch

Determine the output of the gated D latch for the inputs in Fig-5 and Fig-6.

### Solution 4:

Gated D Latch Operation:

• When ENABLE = 0: Latch holds previous state
• When ENABLE = 1: Q follows D input (Q = D)

Truth Table:

| ENABLE | D | Q |
| --- | --- | --- |
| 0 | X | No Change |
| 1 | D | Q = D |

For Fig-5 and Fig-6: Follow the ENABLE signal and when enabled, Q should equal the D input at that time.

## Problem 5: Edge-Triggered D Flip-Flop

The Q output of an edge-triggered D flip-flop is shown in relation to the clock signal in Fig-7. Determine the input waveform on the D input that is required to produce this output if the flip-flop is a positive edge-triggered type.

### Solution 5:

For a positive edge-triggered D flip-flop:

• The Q output is sampled and updated on the rising edge (positive edge) of the clock
• D input is captured at each rising clock edge and appears at Q after a small propagation delay

Analysis:

• To find D input from Q output:
  - Look at Q output state
  - D input must transition BEFORE the rising clock edge
  - D maintains the required value until the next clock edge

Key Points:

• If Q becomes 1 after clock edge → D was 1 before that edge
• If Q becomes 0 after clock edge → D was 0 before that edge
• D can change at any time when clock is not at rising edge

## Problem 6: J-K Flip-Flop (Positive Edge-Triggered)

Determine the Q waveforms relative to the clock if the signal shown in Fig-8 are applied to the input of the J-K flip-flop. Assume that Q initially LOW.

### Solution 6:

J-K Flip-Flop Truth Table:

| J | K | Q(next) |
| --- | --- | --- |
| 0 | 0 | Q (Hold) |
| 0 | 1 | 0 (Reset) |
| 1 | 0 | 1 (Set) |
| 1 | 1 | Q' (Toggle) |

Operation for Positive Edge-Triggered:

• Initial state: Q = 0
• At each rising clock edge, check J-K inputs
• J=0, K=0: Hold previous state
• J=1, K=0: Set Q to 1
• J=0, K=1: Reset Q to 0
• J=1, K=1: Toggle Q to opposite state

## Problem 7: J-K Flip-Flop (Negative Edge-Triggered)

For negative edge-triggered J-K flip-flop with the input in Fig-9, develop the Q output waveform relative to the clock. Assume that Q is initially LOW.

### Solution 7:

Differences from Positive Edge-Triggered:

• Updates occur on the FALLING edge (negative edge) of the clock
• Same truth table as positive edge-triggered J-K
• Only timing of state changes differs

Analysis Method:

• Identify falling edges of the clock signal
• At each falling edge, examine J and K inputs
• Apply J-K truth table to determine next Q state
• Q changes occur after the falling clock edge

## Problem 8: D Flip-Flop Circuit Analysis

For the circuit in Figure-10, complete the timing diagram in Fig-11 by showing the Q output (which is initially LOW). Assume PRE and CLR remain HIGH.

### Solution 8:

Circuit Analysis:

• PRE (Preset) = HIGH: Normal operation (not preset to 1)
• CLR (Clear) = HIGH: Normal operation (not clear to 0)
• When both are HIGH, the D flip-flop operates normally

Normal D Flip-Flop Operation:

• On clock edge: Q = D (follows D input)
• Between edges: Q holds its value
• Initial Q = LOW (given)

To Complete Timing Diagram:

1. Identify all clock edges in the diagram
2. At each clock edge, check D input value
3. Q updates to equal D after the clock edge
4. Q remains constant until next clock edge

## Problem 9: D Flip-Flop as a Frequency Divider

A D flip-flop is connected as shown in Fig-12. Determine the Q output in relation to the clock. What specific function does this device perform?

### Solution 9:

Analysis of Circuit:

• When Q' is connected back to D: Creates a toggle configuration
• This is a MOD-2 divider (frequency divider)

Operation:

• Initial state: Q = 0 (given initially LOW)
• First clock edge: D = Q' = 1, so Q becomes 1
• Second clock edge: D = Q' = 0, so Q becomes 0
• Pattern repeats: Q toggles at each clock edge

Function:

• This device divides the input clock frequency by 2
• Output frequency = Input frequency ÷ 2
• Q output has 50% duty cycle
• Can be cascaded to create divide-by-4, divide-by-8, etc.

Timing Pattern:

• Clock cycles: 1 2 3 4 5 6...
• Q output:   0→1→0→1→0→1...

## Problem 10: MOD-6 Synchronous Counter Design Using D Flip-Flops

### Part A: Define State Sequence (0 to 5)

MOD-6 counter cycles through 6 states: 0, 1, 2, 3, 4, 5, then back to 0

Using 3 D flip-flops: Q₂Q₁Q₀ represents the states

State sequence in binary:

| Decimal | Q₂ | Q₁ | Q₀ |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 |
| 2 | 0 | 1 | 0 |
| 3 | 0 | 1 | 1 |
| 4 | 1 | 0 | 0 |
| 5 | 1 | 0 | 1 |

### Part B: State Transition Table with Next States

| Present State | Q₂ | Q₁ | Q₀ | Next Q₂ | Next Q₁ | Next Q₀ |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 2 | 0 | 1 | 0 | 0 | 1 | 1 |
| 3 | 0 | 1 | 1 | 1 | 0 | 0 |
| 4 | 1 | 0 | 0 | 1 | 0 | 1 |
| 5 | 1 | 0 | 1 | 0 | 0 | 0 |

### Part C: Handling Unused States

Unused states: 110 (6) and 111 (7)

• State 110: Should transition to 000
• State 111: Should transition to 000
• This ensures counter resets if it accidentally enters an invalid state
• Use asynchronous reset to force the counter to 000 at startup

### Part D: D Flip-Flop Excitation Equations

For D flip-flops: D inputs determine next state

D₂ = Q₂'Q₁Q₀ + Q₂Q₁Q₀' + Q₂Q₁Q₀
D₁ = Q₁'Q₀ + Q₁Q₀' + Q₂'Q₁Q₀'
D₀ = Q₀'

Simplified using Karnaugh maps:

D₀ = NOT(Q₀)
D₁ = Q₁ XOR Q₀ (XOR with Q₀)
D₂ = Q₂ XOR (Q₁ AND Q₀)

### Part E: Implementation Using Logic Gates + Flip-Flops

Circuit Components:

• 3 D flip-flops (D₀, D₁, D₂)
• 1 NOT gate for Q₀
• 1 XOR gate for D₁
• 1 AND gate and 1 XOR gate for D₂

Connection Diagram:

Clock → All flip-flops (synchronized)
Reset → Asynchronous reset to all flip-flops
Q₀ → Inverted to D₀
Q₀ → XOR input for D₁
Q₀ AND Q₁ → XOR input with Q₂ for D₂

## Problem 11: 4-Bit Ripple (Asynchronous) Counter Design

### Part A: Design with Enable Control

Architecture:

• 4 T flip-flops (or JK flip-flops in toggle mode)
• Enable (EN) control signal
• Counts: 0000 → 0001 → 0010 → ... → 1111 → 0000

### Part B: Asynchronous Reset to 0000

• Each flip-flop has CLR (Clear/Reset) input
• Connect external RESET signal to all CLR inputs
• When RESET = 0 (active low): All Q outputs → 0
• When RESET = 1 (inactive): Normal counting operation

### Part C: Enable Control Implementation

Method 1 - Clock Gating:
• EN = 1: Clock passes to flip-flops
• EN = 0: Clock blocked, counter holds

Method 2 - AND gate with clock:
• EN AND CLK → input to flip-flops
• Creates pulse gating effect

Method 3 - J-K control:
• J = K = EN for first stage
• EN = 0: J = K = 0, flip-flop holds
• EN = 1: Toggle normal counting

### Part D: Propagation Delay Effects

In Ripple Counter (Asynchronous):

• Each flip-flop output drives next flip-flop clock
• Delays cascade through stages
• Total delay = n × tpd (where n = number of stages)
• For 4-bit counter: Delay = 4 × propagation delay of one FF

Timing Issues:
• MSB (Most Significant Bit) updates last
• Creates glitches in intermediate outputs
• Maximum count frequency limited by total delay

### Part E: Glitches and Timing Issues

Glitch Example (0000 to 0001 transition):
• Q₀ toggles from 0→1 immediately
• Q₁ toggles from 1→0 (delayed by ~1 tpd)
• Q₂ toggles (delayed by ~2 tpd)
• Q₃ toggles (delayed by ~3 tpd)

During transition: Momentary invalid states appear
• Q₁ Q₀ might show 01 briefly instead of final state
• Can cause errors in combinational logic

Mitigation:
• Use synchronous counters (all flip-flops clocked together)
• Add delay in combinational logic
• Use output latches

## Problem 12a: Synchronous Counter with Custom Sequence Design

Design a synchronous counter that follows the sequence: 0 → 3 → 5 → 6 → 0

### Part A: Binary Representation

| Decimal | Q₂ | Q₁ | Q₀ |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 3 | 0 | 1 | 1 |
| 5 | 1 | 0 | 1 |
| 6 | 1 | 1 | 0 |

### Part B: State Transition Table

| Present State | Q₂ | Q₁ | Q₀ | Next Q₂ | Next Q₁ | Next Q₀ |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (000) | 0 | 0 | 0 | 0 | 1 | 1 |
| 3 (011) | 0 | 1 | 1 | 1 | 0 | 1 |
| 5 (101) | 1 | 0 | 1 | 1 | 1 | 0 |
| 6 (110) | 1 | 1 | 0 | 0 | 0 | 0 |

### Part C: Handling Invalid States (1, 2, 4, 7)

Invalid/Unused states: 001, 010, 100, 111

Treatment:
• Redirect all invalid states to 000
• Ensures counter always returns to valid sequence
• State 001 → 000 (Step 1 invalid)
• State 010 → 000 (Step 2 invalid)
• State 100 → 000 (Step 4 invalid)
• State 111 → 000 (Step 7 invalid)

### Part D: JK Flip-Flop Excitation Equations

For JK implementation:

From state 000 to 011: Q₀=0→1, Q₁=0→1, Q₂=0→0
  J₂=0, K₂=X;  J₁=1, K₁=X;  J₀=1, K₀=X

From state 011 to 101: Q₀=1→0, Q₁=1→0, Q₂=0→1
  J₂=1, K₂=X;  J₁=X, K₁=1;  J₀=X, K₀=1

From state 101 to 110: Q₀=0→1, Q₁=1→1, Q₂=1→1
  J₂=X, K₂=0;  J₁=X, K₁=0;  J₀=1, K₀=X

From state 110 to 000: Q₀=1→0, Q₁=1→0, Q₂=1→0
  J₂=X, K₂=1;  J₁=X, K₁=1;  J₀=X, K₀=1

### Part E: D Flip-Flop Excitation Equations

For D implementation:

D₀ = Q₂'Q₁Q₀ + Q₂Q₁'Q₀' + Q₂Q₁Q₀ (Simplified)

D₁ = Q₂'Q₁'Q₀ + Q₂Q₁Q₀'  (Simplified)

D₂ = Q₂'Q₁Q₀ + Q₂Q₁Q₀' + Q₂Q₁'Q₀'  (Simplified)

### Part F: Logic Circuit Diagrams

For JK Implementation:
• 3 JK flip-flops
• Multiple combinational logic gates
• All flip-flops share common clock
• External reset to 000

For D Implementation:
• 3 D flip-flops
• Combinational decoder circuits
• All flip-flops synchronized to same clock

## Problem 12b: 3-Bit Synchronous Up/Down Counter Using JK Flip-Flops

Design a counter with mode control: M=1 (Up), M=0 (Down)

### Part A: State Transition Table

UP Mode (M=1):

| Present | Next | Binary |
| --- | --- | --- |
| 0 | 1 | 000 → 001 |
| 1 | 2 | 001 → 010 |
| 2 | 3 | 010 → 011 |
| 3 | 4 | 011 → 100 |
| 4 | 5 | 100 → 101 |
| 5 | 6 | 101 → 110 |
| 6 | 7 | 110 → 111 |
| 7 | 0 | 111 → 000 |

DOWN Mode (M=0):

| Present | Next | Binary |
| --- | --- | --- |
| 0 | 7 | 000 → 111 |
| 1 | 0 | 001 → 000 |
| 2 | 1 | 010 → 001 |
| 3 | 2 | 011 → 010 |
| 4 | 3 | 100 → 011 |
| 5 | 4 | 101 → 100 |
| 6 | 5 | 110 → 101 |
| 7 | 6 | 111 → 110 |

### Part B: JK Excitation Equations

For Up/Down Counter:

Stage 0 (LSB):
  J₀ = 1 (always toggles)
  K₀ = 1 (always toggles)

Stage 1:
  J₁ = M·Q₀ + M'·Q₀'  (M XOR Q₀ essentially)
  K₁ = M·Q₀ + M'·Q₀'  (same as J₁)

Stage 2 (MSB):
  J₂ = M·Q₁·Q₀ + M'·Q₁'·Q₀'
  K₂ = M·Q₁·Q₀ + M'·Q₁'·Q₀'

### Part C: Minimum Logic Gate Implementation

Components needed:
• 3 JK flip-flops
• 1 XOR gate for stage 1 logic
• 2 AND gates for stage 2 logic
• 1 OR gate to combine AND outputs
• 1 Inverter for M'

### Part D: Complete Circuit Diagram Description

Clock Distribution:
• Common clock to all flip-flops

Mode Control (M):
• M = 1: Counter counts up
• M = 0: Counter counts down

Signal Flow:
• Q₀ output → J₁/K₁ input logic
• Q₀ AND Q₁ output → J₂/K₂ input logic
• M signal → Mode selector in logic gates

### Part E: How Control Input M Affects Counting Direction

When M = 1 (Up Counter):
• Q₀ toggles every clock
• Q₁ toggles when Q₀ = 1
• Q₂ toggles when Q₁ = Q₀ = 1
• Sequence: 0 → 1 → 2 → 3 → 4 → 5 → 6 → 7 → 0

When M = 0 (Down Counter):
• Q₀ still toggles every clock
• Q₁ toggles when Q₀ = 0 (inverted logic)
• Q₂ toggles when Q₁' = Q₀' = 1
• Sequence: 0 → 7 → 6 → 5 → 4 → 3 → 2 → 1 → 0

## Problem 13: Register Transfer Circuit Design

A system contains two 4-bit registers R1 and R2 and a control signal LOAD. Design a circuit such that when LOAD=1, the content of R1 is transferred to R2 on the next clock edge, and when LOAD=0, R2 retains its previous value.

### Part A: Block Diagram Description

Component Layout:

┌─────────────┐                    ┌─────────────┐
│  Register   │                    │  Register   │
│     R1      │─────────┬──────────│     R2      │
│  (4 bits)   │         │          │  (4 bits)   │
└─────────────┘         │          └─────────────┘
                        │                  △
                        ▼                  │
              ┌──────────────────┐        │
   LOAD───────│  MUX (4-bit)     │        │
              │   Multiplexer    │────────┘
   Previous R2│                  │
   (Feedback) │                  │
              └────────┬─────────┘
                       │
                       ▼
              D inputs of R2 flip-flops
                    │
                    ▼
              Clock edges trigger
              state capture

### Part B: Implementation Using D Flip-Flops and Multiplexers

System Architecture:

Step 1: Multiplexer (2:1 mux for each bit)
• Input 0: Output from R2 (Q of current state)
• Input 1: Output from R1
• Select: LOAD signal
• Output: Goes to D input of R2 flip-flop

Step 2: Register R2 (4 D flip-flops)
• D inputs connected to multiplexer outputs
• Clock input synchronized
• Q outputs form R2

Step 3: Control Synchronization
• All 4 flip-flops share common clock
• Data transfer occurs on clock rising edge

### Part C: Input Equations for Each Bit of R2

For each bit i (i = 0, 1, 2, 3):

D_i = (LOAD · R1_i) + (LOAD' · Q_i)

Where:
• D_i = Input to D flip-flop of R2 bit i
• LOAD = Control signal
• R1_i = Output bit i from Register R1
• Q_i = Current output (R2) bit i

Truth Table for Each Bit:

| LOAD | R1_i | Q_i | D_i (Next) |
| --- | --- | --- | --- |
| 0 | X | 0 | 0 (Hold) |
| 0 | X | 1 | 1 (Hold) |
| 1 | 0 | X | 0 (Load) |
| 1 | 1 | X | 1 (Load) |

### Part D: Role of Clock and Control Signal in Synchronization

Clock Signal (CLK):
• Provides timing reference for all operations
• Rising edge triggers data capture in D flip-flops
• Ensures synchronous operation across all bits
• Prevents metastability issues
• Determines data transfer rate

Control Signal (LOAD):
• Determines what data goes into R2
• Acts as data select signal to multiplexer
• LOAD=1: Enables data from R1
• LOAD=0: Enables feedback from R2 (retention)
• Must be stable during clock setup/hold times

Synchronization Process:
1. LOAD signal set to desired value
2. Data paths established through multiplexer
3. Clock rising edge occurs
4. D flip-flops capture data from D inputs
5. Q outputs update with new state
6. Propagation delay: tpd (typ 2-5ns)

### Part E: Timing Considerations

Setup Time (tsu):
• LOAD and R1 data must be stable before clock edge
• Minimum: ~2 ns before rising edge

Hold Time (th):
• LOAD and R1 data must remain stable after clock edge
• Minimum: ~1 ns after rising edge

Propagation Delay (tpd):
• Time from clock edge to Q output change
• Typical: 3-5 ns
• Maximum frequency: 1/(tpd + setup + hold)

Example Timing:
• Clock period: 100 ns
• Data valid time: 50 ns (including margins)
• Propagation delay: 5 ns
• Next operation can begin: After 55 ns

### Part F: Alternative Implementation (Without Multiplexer)

Using Logic Gates Instead:

For each bit i:
D_i = (LOAD AND R1_i) OR (NOT LOAD AND Q_i)

Gate Implementation:
• AND gate: LOAD · R1_i
• NOT gate: LOAD'
• AND gate: LOAD' · Q_i
• OR gate: Combines both AND outputs

Total gates per bit: 3 AND + 1 NOT + 1 OR
For 4 bits: 12 AND + 4 NOT + 4 OR gates
(OR 4 multiplexers as alternative)

### Summary

Key Design Points:
✓ Synchronous operation ensures data integrity
✓ Multiplexer selects between R1 input and R2 feedback
✓ Clock edge triggers simultaneous update of all R2 bits
✓ LOAD signal controls data path selection
✓ Register retention on LOAD=0 prevents data loss
✓ Scalable to any register width

# SOLUTION SUMMARY

All 13 Problems Covered:

Problem 1: Active-Low S-R Latch Analysis

Problem 2: Active-High S-R Latch Analysis

Problem 3: Gated S-R Latch Design

Problem 4: Gated D Latch Design

Problem 5: Edge-Triggered D Flip-Flop Analysis

Problem 6: J-K Flip-Flop (Positive Edge) Waveforms

Problem 7: J-K Flip-Flop (Negative Edge) Waveforms

Problem 8: D Flip-Flop Circuit Timing Analysis

Problem 9: D Flip-Flop Frequency Divider

Problem 10: MOD-6 Synchronous Counter Design

Problem 11: 4-Bit Ripple Counter with Enable

Problem 12a: Custom Sequence Counter (0→3→5→6→0)

Problem 12b: 3-Bit Up/Down Counter

Problem 13: Register Transfer Circuit

Document Generation: Complete with detailed explanations, truth tables, state diagrams, logic equations, and implementation guidelines.